CERN ACADEMIC TRAINING

ENERGY CONCEPTS FOR THE 21ST CENTURY

Particle physics contribution to the elimination of nuclear waste

Jean-Pierre Revol

Progress in particle accelerator technology makes it possible to use a proton accelerator to eliminate nuclear waste efficiently. The Energy Amplifier (EA) proposed by Carlo Rubbia and his group is a subcritical fast neutron system driven by a proton accelerator. It is particularly attractive for destroying, through fission, transuranic elements produced by present nuclear reactors. The EA could also transform efficiently and at minimal cost long-lived fission fragments using the concept of Adiabatic Resonance Crossing (ARC) recently tested at CERN with the TARC experiment. The ARC concept can be extended to several other domains of application (production of radioactive isotopes for medicine and industry, neutron research applications, etc.).

1	Introduction

The research work presented here is an exceptional contribution for a laboratory such as CERN, in principle devoted entirely to fundamental research. However, the Energy Amplifier (EA) [1] is an innovative approach to nuclear energy, and it should come as no surprise that such an innovation results from fundamental research which has always been a main driving engine of innovation. Examples are legion and well known; one of the most recent, the World-Wide Web, was invented at CERN and not by the much more powerful and resourceful computer industry.

Because particle physicists, interested in discovering the ultimate structure of matter, have pushed particle accelerator technology as far as they have, it is possible today to consider using a proton accelerator to drive a new type of nuclear system, with very attractive properties.

Today, the world is facing an extremely difficult challenge, that of producing sufficient energy to sustain economic growth without ruining the ecological equilibrium of the planet. The massive use of fossil fuels has allowed the Western World to reach an unprecedented level of wealth. Unfortunately, if the rest of the Earth's population were to carry out the same energy policy, the entire planet would be in serious trouble. There is, therefore, a moral obligation for developed countries to provide new energy sources for the entire world in order to minimize global warming and other pollution effects.

If an acceptable solution is found, it will certainly be the result of systematic R&D and in this context, nuclear energy should be part of this R&D. The present nuclear energy programme is meeting growing public opposition world-wide because of three main reasons: (a) the association with military use and the fear of nuclear weapon proliferation; (b) the fear of accidents such as Chernobyl (1986 prompt-supercritical reactivity excursion) and Three Mile Island (1979 loss-of-coolant accident resulting in a core meltdown); (c) the issue of the back-end of the fuel cycle (nuclear waste management: at this time only deep geological storage is seriously envisaged).

Obviously, nuclear power, without these drawbacks would be ideal as it does not release greenhouse gases nor other chemical pollutants (NOx, SOx, etc.), nor dust particles, nor even radioactive particles as coal ashes do. Therefore, the real

question facing scientists today is: Is it possible to transform nuclear energy production in such a way as to make it acceptable to society? Nuclear energy is a domain that has essentially seen no significant fundamental R&D since the end of the 1950s when the first civil power plants came into operation. There have been many technological improvements, mainly with the purpose of improving safety. However, we have seen that even these were not sufficient.

The concept of the EA was proposed by C. Rubbia and his group specifically as an answer to the concerns raised by current nuclear energy production. The present EA version is optimized for the elimination of the nuclear waste, as it is considered to be the most pressing issue in the Western World. In developing countries such as China and India, where there is virtually no nuclear waste, a version of the EA optimized for energy production, adapted to the detailed needs of the country and with minimized waste production, is the more appropriate solution. It is interesting to note that the Chinese Government has just approved the first phase of an R&D project on an EA system for energy production.

2 	Nuclear waste

Transuranic elements (TRU) and fission fragments (FF) are the two main components of nuclear waste representing respectively 1.1% and 4% of spent nuclear fuel. TRU, which are produced by neutron capture in the fuel eventually followed by decay, can only be destroyed by fission, while FF can only be destroyed by neutron capture; therefore, different methods will have to be used to eliminate them. As the long term radiotoxicity of waste (Figure 1) is clearly dominated by TRU, the EA has been designed to destroy them with the highest efficiency.

Figure 1: Time evolution of the potential radiotoxicity (relative to uranium ore) of the two main components of nuclear waste for PWR spent fuel, obtained with the ORIGEN2 code.

3 	The Energy Amplifier

The Energy Amplifier is a subcritical system, driven by a proton accelerator and using fast neutrons (Figure 2). A complete description of all the features of the EA can be found in Ref. [1]. One of the main characteristics is the presence of 104 tons of

molten lead used as a target for the protons to produce neutrons by spallation, as a neutron moderator, as a coolant to extract heat by natural convection and as a radioactivity containment medium.

Figure 2: Schematics of the 1500 MWth Energy Amplifier standard unit [1]. The main vessel is about 25 m high and 6 m in diameter. The proton beam is injected vertically, through a vacuum pipe to produce spallation neutrons at the level of the core.

3.1 	Why fast neutrons?

The choice of lead as a neutron moderator to obtain the hardest possible neutron energy spectrum is deliberate. This is dictated by the need to optimize the fission probability of TRU. Indeed, in the fast neutron flux provided by the EA all TRU can undergo fission, a process which eliminates them, while in a PWR thermal neutron flux many TRU do not fission and thus accumulate as waste (Figure 3).

In addition, as the capture cross section of neutrons on FF is smaller for fast neutrons than for thermal neutrons (Figure 4), and since neutron capture on FF is the main limitation to long burnups, in a fast neutron system the efficiency with which the

fuel can be used will be much higher than in a PWR. Typically it is hoped to reach burnups of 150 GW×day/t (a burnup of 200 GW×d/t was achieved in the fast EBR2 system at Argonne National Laboratory).

Figure 3: Comparison of fission and capture probabilities of actinides for thermal and fast neutron fluxes.

Figure 4: Fraction of neutron captures on fission fragments (FF) for thermal and fast neutron fluxes, as a function of burnup. The maximum burnup for a PWR is indicated.

3.2	Subcriticality and the accelerator

The proposed system [1] has a neutron multiplication coefficient (k) of 0.98. The sustainability of the nuclear fission reactions is made possible because of the presence of an external source of neutrons provided by the proton beam. The working point is far below criticality, which ensures that the system remains subcritical at all times, implying that, by construction, accidents of the Chernobyl type are impossible. The traditional keff of the system itself (with beam turned off) is even smaller than k (of the order of 0.97). The energy amplification in the system, defined as the ratio between the energy produced in the EA and the energy provided by the beam, can be parametrized as G0/(1–k), where G0 is a constant characterizing

the spallation process. This aspect of the system has been studied in the FEAT experiment [2] at CERN where it was shown that this energy gain is well understood and that, not only is it independent of the proton beam intensity, but it is also independent of the beam kinetic energy if above about 900 MeV. This fortunate feature means that the accelerator can be of relatively modest size (Figure 5). All experts agree that the present accelerator technology can provide the required beam power (10 to 20 mA at 1 GeV) with either linac or cyclotron solutions [3]. Examples already exist of high power accelerators which are planned or have been considered in various parts of the world:

– the PSI (Switzerland) cyclotron now running at 1.4 mA, 590 MeV, 0.826 MW [4];

– the proton linac for the Los Alamos Neutron Science Centre (LANSCE) running up to 1.5 mA, 0.8 GeV and 1 MW of average power [5];

– both the USA and Europe had projects to build linacs to produce tritium: (TRISPAL at CEA (France): 600 MeV, 40 mA, 24 MW and LANL (USA): 1 GeV, 100 mA, 100 MW). Even though tritium is no longer officially on the agenda, accelerator developments are continuing;

– Japan is also considering a high intensity proton source as part of their new Neutron Science Project [6].

The system needed to drive an EA represents only a reasonable extrapolation of what has already been achieved in current accelerator technology.

Figure 5: The full cyclotron high intensity accelerator layout proposed to drive a k = 0.98 EA [1].

In practice, the choice of accelerator technology may be coupled with the strategy for the EA system. If the strategy is to destroy waste on a nuclear power plant site, then the cyclotron with its smaller size (Figure 5) has a clear advantage (it is easier not to have to extend the power plant site, and easier for control and safety of the accelerator, cost effectiveness).

Several other technical advantages can be found in favour of the cyclotron as compared to a linac:

– One can achieve high efficiency (50%), as the current in RF cavities would be about 100 times (100 turns) the extraction current, implying that most power is in the RF cavities (which have reached 70% efficiency), keeping the losses relatively small.

– There is no need for SC cavities, keeping the technology simple.

– The reliability may be better than in linacs which need many more control elements (reliability decreases strongly with an increase in the number of parts).

– In a warm linac the small aperture is a problem for the beam losses, which in addition are not localized (a superconducting linac is difficult below 400 MeV because of the problem of making short niobium-coated superconducting cavities). In cyclotrons, beam losses are expected to be small and localized. Machine elements are accessible as soon as it comes to a halt (this is the case at PSI).

Figure 6: The energy amplification scheme in the standard EA system as proposed in Ref. [1].

An important achievement of the FEAT experiment was the validation of the innovative simulation developed by the EET group at CERN of energy amplification in accelerator driven subcritical systems. This gives confidence in the choice of the main parameters of a system where less than 5% of the electric power needs to be recirculated during its operation (Figure 6).

3.3	Destruction of nuclear waste: TRU

The general strategy consists of using as fuel thorium mixed with TRU as opposed to uranium with plutonium as proposed in fast critical reactors, such as SuperPhenix.

The availability of an external neutron source, thanks to the accelerator, and the availability of a fast neutron energy spectrum, thanks to the choice of lead as moderator, allows the sustained operation of a subcritical device with wide flexibility in the choice of fuel. Pure thorium does not fission, but it is 233U bred from 232Th which can produce energy through fission. In practice, seeds are needed to provide fissions at the startup of the system, and for this purpose any fissionable element will do: 233U from a previous EA fuel load or 235U extracted from natural uranium or military 239Pu or simply TRU, which is precisely the main component of the waste we wish to destroy. Therefore, it is possible, in an EA, to destroy TRU by fission, a process which produces energy and makes the method economically attractive. TRU represent potentially about 40% of the energy that a PWR delivers while producing these TRU.

Thorium is an attractive fuel because it exists in relatively large quantities in the Earth's crust (at least five times more abundant than uranium), it is isotopically pure (no enrichment is needed), all of it is used in the EA as compared to only 0.7% of 235U in a PWR and it is about 5 neutron captures away from the TRU one wants to destroy, ensuring that it can work in a mode where it destroys more TRU than it produces.

Figure 7a: Net plutonium consumption per unit energy in a uranium-plutonium fast breeder (CAPRA [7]) as a function of plutonium concentration. Note that the unit is kg/TW×h electric and not thermal.

Figure 7b: Evolution as a function of burnup of the stockpile of the main elements present in the EA fuel [1].

It is easy to see why a thorium system would be much more practical than a uranium system for the destruction of TRU. The high equilibrium concentration (15%) of plutonium in uranium type systems (Figure 7a) forces the use of extremely large plutonium enrichment, which would make these systems extremely dangerous, while in an EA, equilibrium concentrations of the order of 10–5 (Figure 7b) naturally ensure a high burning rate for reasonable TRU concentrations.

A study [8] carried out for the Spanish Government, based on a practical example, showed that a 1500 MWth EA could destroy a net amount of 34 kg of TRU per TW×hth of thermal energy produced. In comparison, a PWR, on the contrary, produces 14 kg of TRU per TW×hth.

It is expected that the reprocessing needed to extract TRU from spent fuel should be much simpler than what is needed to extract plutonium from spent fuel for MOX, as performed in the La Hague factory (PUREX process). A pyrolectric reprocessing method [9] developed at the Argonne Laboratory in the United States collects all TRU on a single electrode; this is sufficient since all of them fission and do not need to be separated from one another.

3.4 	Why not a critical system using thorium?

Critical reactors using thorium fuel have worked in the past [10], motivated by the prospect of a high neutron yield per neutron absorbed which 233U offers over the whole neutron energy range, only slightly surpassed by 239Pu for fast neutrons. However, there is a price to pay for breeding 233U. It is the production of 233Pa which has a large neutron capture cross-section and must be compensated by a higher enrichment in fissile material. Also, 233U fissions produce more 135Xe (direct yield 1.4 % for 233U versus 0.3% for 235U) and samarium precursors (147Nd, 149Pm) than 235U. These isotopes represent a significant fraction of the total neutron absorption by fission products. At mid-cycle they account for more than 50% of the total fission product absorption.

Finally, the effective fraction of delayed neutrons (βeff.) of 233U is less than half of that of 235U, leading to a smaller safety margin. This goes against our strategy of a different approach to safety in the choice of a subcritical system. In a critical system, the effective neutron multiplication coefficient (k) is maintained equal to one by active control and feedback. The resulting safety of the system is then defined in terms of the probability for the system to become (or not to become) supercritical (k>1), as happened in Chernobyl in 1986. The probability of such an accident occurring may be very small, but is not zero. In a subcritical system, the effective neutron multiplication coefficient is smaller than one by construction. Therefore, the resulting safety aspect is a deterministic one. The system is and remains subcritical at all times and Chernobyl type accidents are simply impossible.

Furthermore, the availability of an external neutron source in an EA allows greater flexibility in the choice of fuel than in a critical system, particularly relevant for TRU burning.

3.5	Destruction of nuclear waste: Long-Lived Fission Fragments (LLFF)

In a system such as the EA, where TRU are destroyed, the long term (≥ 500 years) radiotoxicity of the waste becomes dominated by LLFF (Figure 8). This residual level of radiotoxicity could perhaps be tolerated, since it is lower than the level of radiotoxicity of coal ashes corresponding to the production of the same quantity of energy. However, since the main LLFF (99Tc and 129I) can be soluble in water and therefore have a non-zero probability over a time-scale of million of years of contaminating the biological chain with hard-to-predict long term effects, it may be wise to destroy them also.

In order to provide such an option, Carlo Rubbia has proposed to use Adiabatic Resonance Crossing (ARC) [12] (Figure 9) to enhance the neutron capture probability, turning for instance a 2.1×105 year half-life 99Tc into 100Tc decaying quickly (t1/2 ~ 15.8 s) into stable 100Ru. The TARC experiment at CERN [13] has shown that one can indeed use the peculiar (small elastic collision length λ ~ 3 cm and small elastic ΔE/E) kinematic of neutrons in pure lead (the most transparent to neutrons of all heavy elements) to maximize the neutron capture probability, making optimum use of prominent resonances in the neutron capture cross-section. Note that 129I and 99Tc which were studied in TARC represent 95% of the LLFF volume. The results from TARC imply that one could actually destroy 99Tc and 129I in the lead in the vicinity of the EA core, where conditions are such that one can destroy about twice as much of these elements as is produced over the same time period in the EA core. This possibility to transmute LLFF in a parasitic mode, around an EA core, may be an additional incentive to eliminate LLFF, a process which, unlike the elimination of TRU producing energy, does not pay.

Figure 8: Evolution of the potential radiotoxicity of nuclear waste for PWR, EA and coal burning power station, showing that in the EA, the long-term radiotoxicity can be 4 orders of magnitude smaller than in a PWR in open cycle and is dominated by LLFF if no further incineration is performed (adapted from Ref. [11]).

Figure 9: Illustration of the Adiabatic Resonance Crossing principle, showing how the presence of lead transforms the spallation neutron energy distribution into a flat flux distribution of slowing down neutrons, with iso-lethargic steps smaller than the width of cross-section resonances where they will be captured with high probability. A sketch of the 334 ton TARC lead volume is also shown.

3.6 Medical applications

A second important application domain of ARC is the production of radioisotopes for medical applications [12]. The same technique (TARC), which is very efficient for destroying fission fragments can also be used to induce any other type of nuclear transmutation (ie. radioisotope production), providing an attractive alternative to production with nuclear reactors. A relatively small system free of all the complications of running a critical nuclear reactor has many advantages, as it would:

– favour local radioisotope production thanks to the small size of the system (activator on the hospital site);

– favour the possibility of using shorter-lived isotopes, resulting in a much smaller dose to the patient [example: 128I (25m) instead of 131I (8j)];

– avoid long (costly) transportation allowing smaller doses at the production site;

– allow flexibility in the choice of neutron source according to need: high intensity accelerators (cyclotron) [industrial production of 99mTc (t1/2 = 6 h) from the decay of 99Mo (t1/2 = 65 h)] to radioactive neutron sources [low activity applications]. In TARC, we successfully tested the idea of using natural molybdenum which contains 24.13% of stable 98Mo to produce 99Mo simply by neutron capture, instead of extracting 99Mo from the spent fuel of a nuclear reactor.

These applications were considered sufficiently important that CERN has now obtained a patent [14] on medical radioisotope production based on ARC.

4	Conclusion

One should not forget that fundamental research is a strong driving force in innovation and that it can lead to potential solutions of some of the most difficult problems facing our society at the beginning of the third millennium. In particular, nuclear energy could make an important contribution to the solution to the energy problem and it would be a mistake to exclude it, a priori, from fundamental R&D.

The Energy Amplifier, based on physics principles well established by dedicated experiments at CERN, is the result of an optimization made possible by the use of an innovative simulation code validated in these experiments (FEAT and TARC).

This experimental programme has generated new applications in various fields: medical applications for which CERN now owns a patent, research with the approved CERN TOF facility [15], and other surprising ideas such as a nuclear engine [16] for deep space exploration.

All of which come as an additional reward for those who have been involved in this project.

References

[1] 	C. Rubbia et al., "Conceptual Design of a Fast Neutron Operated High Power Energy Amplifier", CERN/AT/95-44 (ET), Sept. 29, 1995; see also C. Rubbia, "A High Gain Energy Amplifier Operated with fast Neutrons", AIP Conference Proc. 346, Int. Conf. on ADT Technologies and Applications, Las Vegas, 1994.

[2] 	S. Andriamonje et al., Phys. Lett. B348 (1995) 697-709.

[3] 	S. O. Schriber and P. Mandrillon, "Accelerator Break-Out Group Summary", Proc. Second Int. Conf. on Accelerator Driven Transmutation Technologies and Applications, Kalmar, Sweden, June 3-7, 1996, Vol. 2, p1163.

[4] 	S. Stammbach et al., "The 0.9 MW Proton Beam at PSI and Studies on a 10 MW Cyclotron", Proc. Second Int. Conf. on Accelerator Driven Transmutation Technologies and Applications, Kalmar, Sweden, June 3-7, 1996, Vol. 2, p1163.

[5]	See for instance, S. O. Schriber, "Developments of Linacs for ADTT in the USA", Proc. of the 2nd Int. Conf. on ADTT, Kalmar, June 1996 and LANSCE at the following WEB address: http://www.lansce.lanl.gov.

[6]	M. Mizumoto et al., "Proton Linac Development for Neutron Science Project", in Proc. Third Int. Conf. on Accelerator Driven Transmutation Technologies and Applications, Praha, Czech Republic, June 7-11, 1999.

[7] 	J. Rouault et al., "Physics of Plutonium Burning in Fast Reactors: Impact on Burner Core Design", Proc. ANS Topical Meeting on Advances in Reactor Physics, Knoxville, 11-15 April 1994.

[8] 	C. Rubbia, S. Buono, Y. Kadi and J.A. Rubio, "Fast Neutron Incineration in the Energy Amplifier as Alternative to Geological Storage: the Case of Spain", CERN/LHC/97-01 (EET).

[9] 	T. Hijikata et al., "Development of Pyrometallurgical Partitioning of Actinides from High-Level Radioactive Waste", Proc. of Global '93, published by the American Nuclear Society, Inc., Vol. 2, p1074.

[10]	Elk River Reactor: 58 MWth Demonstration Boiling Water Reactor, using 232ThO2-235U, (1962-1968) stopped because of corrosion cracking in the vessel's walls;

Indian Point No 1 Reactor: 270 MWel. PWR, using 232ThO2-235U, stopped because 95% enriched uranium was too expensive;

High Temperature Gas Cooled Reactor, 40 MWel. using 232ThO2-235U, at Peach Bottom, Pa, USA (1966-1974);

Light Water Breeder Reactor, 60 MWel. using 232Th-233U, Shippingport, Pa, USA (shut down in 1982 after reaching a burnup of 60 GWd/t).

[11] 	C. Rubbia, "A Comparison of the Safety and Environmental Advantages of the Energy Amplifier and of Magnetic Confinement Fusion", CERN/AT/96-58 (ET), Dec. 29, 1995.

[12] 	C. Rubbia, "Resonance Enhanced Neutron Captures for Element Activation and Waste Transmutation", CERN/LHC/97-04 (EET), June 22, 1997.

[13] 	H. Arnould et al., Phys. Lett. B458 (1999) 167-180;

H. Arnould et al., "Neutron-Driven Nuclear Transmutation by Adiabatic Resonance Crossing", CERN-SL-99-036 EET, July 26, 1999 and Report to the European Union, DGXII, EUR 19117 EN.

[14]	Neutron Driven Element Transmuter, PCT/EP97/03218.

[15] 	C. Rubbia et al., "A High Resolution Spallation Driven Facility at the CERN-PS to Measure Neutron Cross Sections in the Interval from 1 eV to 250 MeV", CERN/LHC/98-02 (EET), May 30, 1998; "A High Resolution Spallation Driven Facility at the CERN-PS to Measure Neutron Cross Sections in the Interval from 1 eV to 250 MeV: A Relative Performance Assessment", CERN/LHC/98-02 (EET)-add. 1, June 15, 1998;

S. Abramovich et al., "Proposal for a Neutron Time of Flight Facility", CERN/SPSC 99-8, SPSC/P 310, 17 March 1999.

[16] 	C. Rubbia, "Report of the Working Group on a Preliminary Assessment of a New Fission Fragment Heated Propulsion Concept and its Applicability to Manned Missions to the Planet Mars", Italian Space Agency, Rome, April 1, 1999 and private communication.